# Import necessary libraries and modules for the implementation

import sys  # System-specific parameters and functions

import socket  # Networking interface for socket programming

import threading  # Thread-based parallelism

import random  # Random number generation

import numpy as np  # Numerical operations on arrays, including Q-table

import json  # JSON encoding and decoding

import gym  # OpenAI Gym for creating RL environments

from gym import spaces  # Spaces module to define action and observation spaces

import os  # Operating system interfaces

import subprocess  # To execute system commands (e.g., adjusting data rate)

# Define beta value used in vehicle density adjustment formula

beta = 3  # Sensitivity parameter for how vehicle density changes with power transmission

# Ensure SUMO_HOME is set, necessary for controlling the SUMO traffic simulation tool

if 'SUMO_HOME' not in os.environ:

os.environ['SUMO_HOME'] = "/usr/share/sumo"  # Set SUMO_HOME to a default path if not already set

sys.path.append(os.path.join(os.environ['SUMO_HOME'], 'tools'))  # Add SUMO tools to the system path

import traci  # Import TraCI to control SUMO from Python

# Define MCS tables (Modulation and Coding Scheme) for different IEEE 802.11 standards, including 802.11bd

# These tables map the MCS index to the modulation type and data rate

mcs_tables = {

'802.11a': [

('BPSK', 6.0), ('BPSK', 9.0),

('QPSK', 12.0), ('QPSK', 18.0),

('16-QAM', 24.0), ('16-QAM', 36.0),

('64-QAM', 48.0), ('64-QAM', 54.0)

],

'802.11b': [

('DSSS', 1.0), ('DSSS', 2.0),

('CCK', 5.5), ('CCK', 11.0)

],

'802.11g': [

('BPSK', 6.0), ('BPSK', 9.0),

('QPSK', 12.0), ('QPSK', 18.0),

('16-QAM', 24.0), ('16-QAM', 36.0),

('64-QAM', 48.0), ('64-QAM', 54.0)

],

'802.11n': [

(0, 'BPSK', 6.5, 13.5),

(1, 'QPSK', 13.0, 27.0),

(2, 'QPSK', 19.5, 40.5),

(3, '16-QAM', 26.0, 54.0),

(4, '16-QAM', 39.0, 81.0),

(5, '64-QAM', 52.0, 108.0),

(6, '64-QAM', 58.5, 121.5),

(7, '64-QAM', 65.0, 135.0)

],

'802.11ac': [

(0, 'BPSK', 6.5, 13.5, 29.3, 58.5),

(1, 'QPSK', 13.0, 27.0, 58.5, 117.0),

(2, 'QPSK', 19.5, 40.5, 87.8, 175.5),

(3, '16-QAM', 26.0, 54.0, 117.0, 234.0),

(4, '16-QAM', 39.0, 81.0, 175.5, 351.0),

(5, '64-QAM', 52.0, 108.0, 234.0, 468.0),

(6, '64-QAM', 58.5, 121.5, 263.3, 526.5),

(7, '64-QAM', 65.0, 135.0, 292.5, 585.0),

(8, '256-QAM', 78.0, 162.0, 351.0, 702.0),

(9, '256-QAM', None, None, 390.0, 780.0)

],

'802.11ax': [

(0, 'BPSK', 7.3, 15.0, 30.0, 60.0),

(1, 'QPSK', 14.6, 30.0, 60.0, 120.0),

(2, 'QPSK', 21.9, 45.0, 90.0, 180.0),

(3, '16-QAM', 29.2, 60.0, 120.0, 240.0),

(4, '16-QAM', 43.9, 90.0, 180.0, 360.0),

(5, '64-QAM', 58.5, 120.0, 240.0, 480.0),

(6, '64-QAM', 65.8, 135.0, 270.0, 540.0),

(7, '64-QAM', 73.1, 150.0, 300.0, 600.0),

(8, '256-QAM', 87.8, 180.0, 360.0, 720.0),

(9, '256-QAM', 97.5, 200.0, 400.0, 800.0)

],

'802.11p': [

('BPSK', 3.0), ('BPSK', 6.0),

('QPSK', 12.0), ('QPSK', 18.0),

('16-QAM', 24.0), ('16-QAM', 36.0),

('64-QAM', 48.0), ('64-QAM', 54.0)

],

'802.11bd': [

# Example MCS table for 802.11bd

(0, 'BPSK', 7.0, 14.0),  # MCS 0

(1, 'QPSK', 14.0, 28.0),  # MCS 1

(2, 'QPSK', 21.0, 42.0),  # MCS 2

(3, '16-QAM', 28.0, 56.0),  # MCS 3

(4, '16-QAM', 42.0, 84.0),  # MCS 4

(5, '64-QAM', 56.0, 112.0),  # MCS 5

(6, '64-QAM', 63.0, 126.0),  # MCS 6

(7, '64-QAM', 70.0, 140.0),  # MCS 7

(8, '256-QAM', 84.0, 168.0),  # MCS 8

(9, '256-QAM', 105.0, 210.0)  # MCS 9

]

}

# Function to get modulation and data rate based on IEEE standard and bandwidth

def get_modulation_and_datarate(ieee_standard, mcs_index, bandwidth):

# Fetch the appropriate MCS table based on the IEEE standard

mcs_table = mcs_tables.get(ieee_standard)

if not mcs_table:

# Print an error message if the MCS table for the specified standard is not found

print(f"No MCS table found for the defined standard {ieee_standard}.")

return 'N/A', 'N/A'

modulation = None

datarate = None

# For standards with variable rates based on bandwidth

if ieee_standard in ['802.11n', '802.11ac', '802.11ax', '802.11bd']:

for entry in mcs_table:

if entry[0] == mcs_index:

modulation = entry[1]

datarate = entry[2] if bandwidth == 20 else entry[3]

break

else:

# For legacy standards with fixed rates

modulation, datarate = mcs_table[mcs_index]

return modulation, datarate

# Function to adjust data rate using 'iw' command

def adjust_data_rate(interface, datarate):

try:

# Use the 'iw' command to set the data rate on the specified wireless interface

subprocess.check_output(['iw', 'dev', interface, 'set', 'bitrate', f'{datarate}Mbps'])

print(f"Data rate set to {datarate} Mbps on interface {interface}")

except subprocess.CalledProcessError as e:

# Handle any errors that occur during the command execution

print(f"Failed to set data rate: {e}")

# Define the VANET (Vehicular Ad-Hoc Network) environment class using PPO (Proximal Policy Optimization)

class VANETEnv(gym.Env):

def __init__(self, car_logs, ieee_standard='802.11p', bandwidth=20):

super(VANETEnv, self).__init__()

# Define the action space as a continuous value representing the change in power transmission

self.action_space = spaces.Box(low=np.array([-3]), high=np.array([3]), dtype=np.float32)  # Actions: [delta_power]

# Define the observation space as continuous values representing the power transmission and vehicle density

self.observation_space = spaces.Box(low=np.array([1, 0.001]), high=np.array([30, 0.8]), dtype=np.float32)  # Observations: [power_transmission, vehicle_density]

# Store car logs and initialize environment parameters

self.car_logs = car_logs

self.current_state = None

# Set environment constants

self.CBR_target = 0.6  # Target Carrier-to-Interference Ratio

self.safety_distance = 100  # Safety distance in meters

self.path_loss_exponent = 2.5  # Path loss exponent for signal attenuation

self.sensitivity = -92  # Sensitivity of the receiver in dBm

self.message_size = 536  # Message size in bytes

# IEEE standard and bandwidth to be used

self.ieee_standard = ieee_standard

self.bandwidth = bandwidth

def reset(self):

# Randomly select a car log and initialize the state

log = random.choice(self.car_logs)

self.current_state = np.array([log['power_transmission'], log['vehicle_density']])

return self.current_state

def step(self, action):

# Extract current state variables: power transmission (p) and vehicle density (rho)

p, rho = self.current_state

delta_p = action[0]  # Extract the action (change in power transmission)

# Apply the action to the current state, ensuring power stays within the valid range [1, 30]

new_p = np.clip(p + delta_p, 1, 30)

# Recalculate vehicle density based on the new power transmission using the beta value

rho = rho * ((p / new_p) ** (1 / beta))

self.current_state = np.array([new_p, rho])  # Update the state with new values

# Assume CBR is provided by the VANET simulation script and is passed in the log

cbr = self.current_state['cbr']

# Compute the reward based on the updated state

reward = self.reward_function(cbr, new_p, p)

done = True  # Each step ends the episode in this simplified example

return self.current_state, reward, done, {}  # Return the new state, reward, and done flag

def reward_function(self, cbr, p, p_prime):

"""Calculate the reward based on the current CBR and power change."""

def H(x):

return 1 if x >= 0 else 0  # Heaviside function: Returns 1 if x >= 0, else 0

def g(x, k):

return x * (H(x) - 2 * H(x - k))  # Custom function to calculate reward components

# Reward function: Combines rewards/penalties based on CBR, power change, and predefined weights

reward = (self.pi_b * g(cbr, self.kb)) - (self.pi_p_prime * abs(p - p_prime)) - (self.pi_p2 * g(p_prime, 20))

# Additional reward or penalty based on proximity to the target CBR

if abs(cbr - self.CBR_target) <= 0.025:

reward += 10  # Bonus for being close to the target

else:

reward -= 0.1  # Penalty for deviating from the target

return reward  # Return the calculated reward

# Define a PPO agent class that interacts with the VANET environment

class PPOAgent:

def __init__(self, env, learning_rate=0.0003, gamma=0.99, clip_ratio=0.2, hidden_layers=[64, 64]):

self.env = env  # The environment the agent will interact with

self.gamma = gamma  # Discount factor for future rewards

self.clip_ratio = clip_ratio  # Clipping ratio for policy updates

# Initialize policy and value function parameters

self.policy_weights = [np.random.randn(env.observation_space.shape[0], env.action_space.shape[0]) / np.sqrt(env.observation_space.shape[0])]

self.value_weights = np.random.randn(env.observation_space.shape[0]) / np.sqrt(env.observation_space.shape[0])

# Learning rates for updating policy and value function

self.policy_lr = learning_rate

self.value_lr = learning_rate

def select_action(self, state):

"""Select an action based on the current policy."""

# Calculate the mean action using the policy weights

mean_action = np.dot(state, self.policy_weights[0])

# Add noise to the action to encourage exploration

action = np.random.normal(mean_action, self.clip_ratio)

# Ensure the action stays within the valid action space range

return np.clip(action, self.env.action_space.low, self.env.action_space.high)

def train(self, num_episodes=10000, batch_size=128):

all_rewards = []  # Store total rewards for each episode

for episode in range(num_episodes):

state = self.env.reset()  # Reset the environment at the start of each episode

rewards, actions, states, next_states = [], [], [], []

done = False

while not done:

action = self.select_action(state)  # Select an action based on the current state

next_state, reward, done, _ = self.env.step(action)  # Take the action and observe the result

# Store the experience (state, action, reward, next state)

states.append(state)

actions.append(action)

rewards.append(reward)

next_states.append(next_state)

state = next_state  # Update the current state

# Perform batch updates if enough experiences have been collected

if len(rewards) >= batch_size:

self.update(states, actions, rewards, next_states)

states, actions, rewards, next_states = [], [], [], []

total_reward = np.sum(rewards)  # Calculate the total reward for this episode

all_rewards.append(total_reward)

# Print progress every 100 episodes

if episode % 100 == 0:

print(f"Episode {episode}: Total Reward: {total_reward}")

# Plot the total rewards over episodes

plt.plot(all_rewards)

plt.xlabel('Episodes')

plt.ylabel('Total Reward')

plt.show()

def update(self, states, actions, rewards, next_states):

# Convert lists to numpy arrays for vectorized operations

states = np.array(states)

actions = np.array(actions)

rewards = np.array(rewards)

next_states = np.array(next_states)

# Compute discounted rewards

discounted_rewards = np.zeros_like(rewards)

running_add = 0

for t in reversed(range(0, len(rewards))):

running_add = running_add * self.gamma + rewards[t]

discounted_rewards[t] = running_add

# Normalize the discounted rewards to stabilize training

discounted_rewards = (discounted_rewards - np.mean(discounted_rewards)) / (np.std(discounted_rewards) + 1e-10)

# Calculate advantages (difference between discounted rewards and value estimates)

values = np.dot(states, self.value_weights)

advantages = discounted_rewards - values

# Update the policy weights using the advantages

for i, state in enumerate(states):

grad = advantages[i] * (actions[i] - np.dot(state, self.policy_weights[0]))

self.policy_weights[0] += self.policy_lr * np.outer(state, grad)

# Update the value function weights using the advantages

for i, state in enumerate(states):

grad_value = advantages[i]

self.value_weights += self.value_lr * grad_value * state

# Define the server-side class to receive car logs and apply PPO optimization

class Server:

def __init__(self, host='localhost', port=12345):

self.host = host  # Server hostname or IP address

self.port = port  # Server port number

self.car_logs = []  # List to store car logs received from clients

self.net = None  # Placeholder for the Mininet object, used for network emulation

def start_server(self):

"""Start the server to receive car logs."""

# Create a TCP/IP socket

server_socket = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

# Bind the socket to the host and port

server_socket.bind((self.host, self.port))

# Listen for incoming connections (up to 5 simultaneous connections)

server_socket.listen(5)

print(f"Server listening on {self.host}:{self.port}...")

# Continuously accept and handle incoming connections

while True:

# Accept a new client connection

client_socket, client_address = server_socket.accept()

print(f"Connection from {client_address}")

# Handle the client connection in a new thread to allow concurrent connections

threading.Thread(target=self.handle_client, args=(client_socket,)).start()

def handle_client(self, client_socket):

"""Handle incoming car logs from the client."""

try:

# Receive data from the client (up to 4096 bytes)

data = client_socket.recv(4096)

if not data:  # If no data received, return early

return

# Decode the received data from JSON format

decoded_data = json.loads(data.decode('utf-8'))

# Add the decoded car log to the list of car logs

self.car_logs.append(decoded_data)

# Process the car log to optimize its settings using PPO

self.process_car_log(decoded_data)

except Exception as e:

print(f"Error handling client: {e}")  # Print any errors that occur

finally:

client_socket.close()  # Ensure the client socket is closed after handling

def process_car_log(self, car_log):

"""Process a car log by optimizing its settings using PPO."""

# Extract relevant parameters from the car log

ieee_standard = car_log.get('ieee_standard', '802.11p')

bandwidth = car_log.get('bandwidth', 20)

mcs_index = car_log.get('mcs_index', 0)

# Initialize the VANET environment and PPO agent

env = VANETEnv([car_log], ieee_standard=ieee_standard, bandwidth=bandwidth)

agent = PPOAgent(env)

agent.train(num_episodes=1)  # Train the agent for one episode

# Select the optimized action (power transmission) based on the current state

new_power_transmission = agent.select_action(env.current_state)[0]

# Get the modulation and data rate based on the IEEE standard, MCS index, and bandwidth

modulation, data_rate = get_modulation_and_datarate(ieee_standard, mcs_index, bandwidth)

# Update the current state in the environment with the new power transmission and vehicle density

env.current_state = [new_power_transmission, car_log['vehicle_density']]

print(f"[INFO] Car {car_log['car_id']}: Adjusted Power Transmission to {new_power_transmission}, Data Rate = {data_rate}")

# Apply the optimized settings to the car in the Mininet simulation

self.apply_optimized_settings(car_log['car_id'], new_power_transmission, data_rate)

def apply_optimized_settings(self, car_id, power_transmission, data_rate):

"""Apply the optimized power transmission and data rate to the car in the simulation."""

try:

# Access the car object in Mininet by car_id

car = self.net.getNodeByName(car_id)

if car:

# Apply the optimized power transmission and data rate to the car

car.setParam("txpower", power_transmission)

adjust_data_rate(car.wintfs[0].name, data_rate)

print(f"Applied settings to {car_id}: Power Transmission = {power_transmission}, Data Rate = {data_rate}")

except Exception as e:

print(f"Error applying settings to {car_id}: {e}")  # Print any errors that occur

def main():

# Initialize and start the server

server = Server(host='localhost', port=9999)

server.start_server()  # Start the server to listen for incoming car logs

# Entry point of the script

if __name__ == "__main__":

main()  # Call the main function to start the server