import sys

import os

import math

import time

import subprocess

import csv

import socket

from threading import Thread

from scapy.all import sniff, Dot11

import re

from scipy.special import gamma

# Ensure SUMO_HOME is set

if 'SUMO_HOME' not in os.environ:

os.environ['SUMO_HOME'] = "/usr/share/sumo"

sys.path.append(os.path.join(os.environ['SUMO_HOME'], 'tools'))

import pytz

import datetime

import traci

from mininet.log import setLogLevel, info

from mn_wifi.cli import CLI

from mn_wifi.net import Mininet_wifi

from mn_wifi.sumo.runner import sumo

from mn_wifi.link import wmediumd, mesh

from mn_wifi.wmediumdConnector import interference

from mn_wifi.node import Car

# MCS tables for different IEEE 802.11 standards, including 802.11bd

mcs_tables = {

'802.11a': [

('BPSK', 6.0), ('BPSK', 9.0),

('QPSK', 12.0), ('QPSK', 18.0),

('16-QAM', 24.0), ('16-QAM', 36.0),

('64-QAM', 48.0), ('64-QAM', 54.0)

],

'802.11b': [

('DSSS', 1.0), ('DSSS', 2.0),

('CCK', 5.5), ('CCK', 11.0)

],

'802.11g': [

('BPSK', 6.0), ('BPSK', 9.0),

('QPSK', 12.0), ('QPSK', 18.0),

('16-QAM', 24.0), ('16-QAM', 36.0),

('64-QAM', 48.0), ('64-QAM', 54.0)

],

'802.11n': [

(0, 'BPSK', 6.5, 13.5),

(1, 'QPSK', 13.0, 27.0),

(2, 'QPSK', 19.5, 40.5),

(3, '16-QAM', 26.0, 54.0),

(4, '16-QAM', 39.0, 81.0),

(5, '64-QAM', 52.0, 108.0),

(6, '64-QAM', 58.5, 121.5),

(7, '64-QAM', 65.0, 135.0)

],

'802.11ac': [

(0, 'BPSK', 6.5, 13.5, 29.3, 58.5),

(1, 'QPSK', 13.0, 27.0, 58.5, 117.0),

(2, 'QPSK', 19.5, 40.5, 87.8, 175.5),

(3, '16-QAM', 26.0, 54.0, 117.0, 234.0),

(4, '16-QAM', 39.0, 81.0, 175.5, 351.0),

(5, '64-QAM', 52.0, 108.0, 234.0, 468.0),

(6, '64-QAM', 58.5, 121.5, 263.3, 526.5),

(7, '64-QAM', 65.0, 135.0, 292.5, 585.0),

(8, '256-QAM', 78.0, 162.0, 351.0, 702.0),

(9, '256-QAM', None, None, 390.0, 780.0)

],

'802.11ax': [

(0, 'BPSK', 7.3, 15.0, 30.0, 60.0),

(1, 'QPSK', 14.6, 30.0, 60.0, 120.0),

(2, 'QPSK', 21.9, 45.0, 90.0, 180.0),

(3, '16-QAM', 29.2, 60.0, 120.0, 240.0),

(4, '16-QAM', 43.9, 90.0, 180.0, 360.0),

(5, '64-QAM', 58.5, 120.0, 240.0, 480.0),

(6, '64-QAM', 65.8, 135.0, 270.0, 540.0),

(7, '64-QAM', 73.1, 150.0, 300.0, 600.0),

(8, '256-QAM', 87.8, 180.0, 360.0, 720.0),

(9, '256-QAM', 97.5, 200.0, 400.0, 800.0)

],

'802.11p': [

('BPSK', 3.0), ('BPSK', 6.0),

('QPSK', 12.0), ('QPSK', 18.0),

('16-QAM', 24.0), ('16-QAM', 36.0),

('64-QAM', 48.0), ('64-QAM', 54.0)

],

'802.11bd': [

# Example MCS table for 802.11bd (values are illustrative)

(0, 'BPSK', 7.0, 14.0),  # MCS 0

(1, 'QPSK', 14.0, 28.0),  # MCS 1

(2, 'QPSK', 21.0, 42.0),  # MCS 2

(3, '16-QAM', 28.0, 56.0),  # MCS 3

(4, '16-QAM', 42.0, 84.0),  # MCS 4

(5, '64-QAM', 56.0, 112.0),  # MCS 5

(6, '64-QAM', 63.0, 126.0),  # MCS 6

(7, '64-QAM', 70.0, 140.0),  # MCS 7

(8, '256-QAM', 84.0, 168.0),  # MCS 8

(9, '256-QAM', 105.0, 210.0)  # MCS 9

]

}

def interface_exists(interface):

try:

subprocess.check_output(['ip', 'link', 'show', interface], stderr=subprocess.STDOUT)

return True

except subprocess.CalledProcessError:

return False

def get_wireless_info(interface, retries=3, delay=2):

for attempt in range(retries):

if interface_exists(interface):

try:

result = subprocess.check_output(['iw', 'dev', interface, 'link'], text=True)

return result

except subprocess.CalledProcessError as e:

print(f"Attempt {attempt+1}: Error accessing interface {interface}: {e}")

time.sleep(delay)

else:

print(f"Attempt {attempt+1}: Interface {interface} does not exist.")

time.sleep(delay)

return None

def parse_wireless_standard(iw_output):

"""Parse the wireless standard from iw command output."""

standard = None

if "phy802.11n" in iw_output:

standard = '802.11n'

elif "phy802.11ac" in iw_output:

standard = '802.11ac'

elif "phy802.11ax" in iw_output:

standard = '802.11ax'

elif "phy802.11p" in iw_output:

standard = '802.11p'

elif "phy802.11a" in iw_output:

standard = '802.11a'

elif "phy802.11b" in iw_output:

standard = '802.11b'

elif "phy802.11g" in iw_output:

standard = '802.11g'

elif "phy802.11bd" in iw_output:  # Add detection for 802.11bd

standard = '802.11bd'

return standard

def parse_mcs_index(iw_output):

"""Parse the MCS index from iw command output."""

mcs_index = None

match = re.search(r'MCS index:\s+(\d+)', iw_output)

if match:

mcs_index = int(match.group(1))

else:

mcs_index = 7  # Default MCS index to 7 for demonstration if not explicitly found

return mcs_index

def get_modulation_and_datarate(interface, ieee_standard):

mcs_table = mcs_tables.get(ieee_standard)

if not mcs_table:

print(f"No MCS table found for the defined standard {ieee_standard}.")

return 'N/A', 'N/A'

iw_output = get_wireless_info(interface)

if not iw_output:

print(f"Unable to get wireless information for {interface}.")

return 'N/A', 'N/A'

mcs_index = parse_mcs_index(iw_output)

modulation = None

datarate = None

# Assuming a 20 MHz bandwidth for simplicity

bandwidth = 20

if ieee_standard in ['802.11n', '802.11ac', '802.11ax', '802.11bd']:  # Added 802.11bd

for entry in mcs_table:

if entry[0] == mcs_index:

modulation = entry[1]

datarate = entry[2] if bandwidth == 20 else entry[3]

break

else:

modulation, datarate = mcs_table[mcs_index]  # Legacy standards have fixed rates

if modulation and datarate:

return modulation, datarate

else:

return 'N/A', 'N/A'

def getdatetime():

"""Get current date and time formatted."""

utc_now = pytz.utc.localize(datetime.datetime.utcnow())

currentDT = utc_now.astimezone(pytz.timezone("Asia/Jakarta"))

DATIME = currentDT.strftime("%Y-%m-%d %H:%M:%S")

return DATIME

def calculate_subnet_mask(number_of_nodes):

"""Calculate subnet mask based on the number of nodes."""

number_of_ips_needed = number_of_nodes + 2

bits_needed_for_hosts = math.ceil(math.log2(number_of_ips_needed))

subnet_mask = 32 - bits_needed_for_hosts

return subnet_mask

def calculate_communication_range(tx_power_dbm, frequency_mhz=2400, threshold_dbm=-90):

"""

Calculate the communication range based on the transmission power using the Free Space Path Loss (FSPL) model.

:param tx_power_dbm: Transmission power in dBm.

:param frequency_mhz: Frequency in MHz (default is 2400 MHz for 2.4 GHz Wi-Fi).

:param threshold_dbm: Receive sensitivity threshold in dBm (default is -90 dBm).

:return: Communication range in meters.

"""

tx_power_watts = 10 ** (tx_power_dbm / 10) / 1000

threshold_watts = 10 ** (threshold_dbm / 10) / 1000

lambda_m = 300 / frequency_mhz

range_m = (lambda_m / (4 * math.pi)) * math.sqrt(tx_power_watts / threshold_watts)

return range_m

def is_within_reach(car1, car2):

"""Check if two cars are within the communication range of each other based on transmission power."""

x1, y1 = car1.params.get('position', (0, 0))

x2, y2 = car2.params.get('position', (0, 0))

distance = math.sqrt((x1 - x2) ** 2 + (y1 - y2) ** 2)

tx_power_car1 = car1.wintfs[0].txpower

tx_power_car2 = car2.wintfs[0].txpower

range_car1 = calculate_communication_range(tx_power_car1)

range_car2 = calculate_communication_range(tx_power_car2)

return distance <= range_car1 and distance <= range_car2

def start_tcpdump(pcap_file):

"""Start tcpdump to capture packets."""

cmd = ["tcpdump", "-i", "any", "-w", pcap_file]

info(f"Starting tcpdump with output file {pcap_file}\n")

process = subprocess.Popen(cmd, stdout=subprocess.DEVNULL, stderr=subprocess.DEVNULL)

return process

def stop_tcpdump(process):

"""Stop tcpdump process."""

if process:

process.terminate()

def calculate_r_cs(S, p, beta, m, wavelength):

A = ((4 * math.pi) / wavelength) ** 2

r_cs = gamma(m + 1/beta) / (gamma(m) * (S * A * (m/p)) ** (1/beta))

return r_cs

def calculate_C(c_d, b_st, n, t_ps):

# Correctly implementing the C formula based on the provided equation

C = (c_d * math.ceil((b_st + n) / c_d) + t_ps) ** -1

return C

def calculate_CBR(r_cs, rho, b, C):

CBR = (2 * r_cs * rho * b) / C

return CBR

class CustomCar(Car):

def __init__(self, name, **params):

super().__init__(name, **params)

self.neighbors_info = {}

self.modulation = None

self.data_rate = None

self.beacon_rate = None

def update_neighbor_info(self, neighbor_id, info):

self.neighbors_info[neighbor_id] = info

def find_neighbors_position(car, net, check_range):

"""Find neighbors of a car based on position."""

neighbors = []

for target_car in net.cars:

if car != target_car and (not check_range or is_within_reach(car, target_car)):

neighbors.append(target_car.name)

return neighbors

def find_neighbors_beacon(car, net, check_range):

"""Find neighbors of a car based on beacon replies."""

neighbors = []

for target_car in net.cars:

if car != target_car and (not check_range or is_within_reach(car, target_car)):

neighbors.append(target_car.name)

return neighbors

def send_beacon(car, beacon_rate, net, check_range, range_detection_method, ieee_standard):

"""Send beacon packets at the specified rate with range checking."""

if range_detection_method == 'position':

neighbors = find_neighbors_position(car, net, check_range)

elif range_detection_method == 'beacon':

neighbors = find_neighbors_beacon(car, net, check_range)

modulation, data_rate = get_modulation_and_datarate(car.wintfs[0].name, ieee_standard)

car.modulation = modulation

car.data_rate = data_rate

car.beacon_rate = beacon_rate

beacon_info = {

'timestamp': getdatetime(),

'car_id': car.name,

'position': car.params.get('position', (0, 0)),  # Use .get to avoid KeyError

'speed': car.params.get('speed', 0),  # Default speed to 0 if not set

'power_transmission': car.wintfs[0].txpower,

'modulation': modulation,

'data_rate': data_rate,

'beacon_rate': beacon_rate,

'channel_busy_rate': car.cbr

}

# Broadcast beacon to neighbors

for neighbor in neighbors:

target_car = net.getNodeByName(neighbor)

target_car.update_neighbor_info(car.name, beacon_info)

def receive_beacon(car, beacon_info):

"""Receive beacon information from a neighbor car."""

neighbor_id = beacon_info['car_id']

car.update_neighbor_info(neighbor_id, beacon_info)

def ping_flood(source, target_ip):

"""Send a flood of ping packets."""

cmd = ["ping", "-i", "0.01", target_ip]

result = subprocess.run(cmd, stdout=subprocess.DEVNULL, stderr=subprocess.DEVNULL)

if result.returncode == 0:

info(f"Successfully sent ping flood from {source.name} to {target_ip}")

def flood_all(net, flood_type, rate_mbps=1, unicast=True, check_range=True):

"""Perform flooding attack between cars."""

def flood_car(car1, car2):

target_ip = car2.params['ip']

if flood_type == 'ping':

ping_flood(car1, target_ip)

threads = []

for car1 in net.cars:

for car2 in net.cars:

if car1 != car2 and (not check_range or is_within_reach(car1, car2)):

thread = Thread(target=flood_car, args=(car1, car2))

threads.append(thread)

thread.start()

for thread in threads:

thread.join()

def setup_socket_connection(server_ip, server_port):

"""Setup a socket connection to the server."""

socket_client = None

try:

socket_client = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

socket_client.connect((server_ip, server_port))

except Exception as e:

print(f"[ERROR] Unable to connect to socket server: {str(e)}")

socket_client = None

return socket_client

def log_data(timestamp, car, log_options, csv_writer=None, socket_client=None):

"""Log data for each car. Supports multiple logging options."""

car_data = {

'timestamp': timestamp,

'car_id': car.name,

'position': car.params.get('position', (0, 0)),

'speed': car.params.get('speed', 0),

'power_transmission': car.wintfs[0].txpower,

'modulation': car.modulation,

'data_rate': car.data_rate,

'beacon_rate': car.beacon_rate,

'channel_busy_rate': car.cbr

}

# Log to CSV if selected

if 'csv' in log_options and csv_writer:

csv_writer.writerow(car_data)

# Log to stdout if selected

if 'stdout' in log_options:

print(car_data)

# Log to socket if selected

if 'socket' in log_options and socket_client:

socket_client.sendall(str(car_data).encode('utf-8'))

for neighbor_id, neighbor_info in car.neighbors_info.items():

neighbor_data = {

'timestamp': timestamp,

'car_id': car.name,

'neighbor_id': neighbor_id,

'position': neighbor_info['position'],

'speed': neighbor_info['speed'],

'power_transmission': neighbor_info['power_transmission'],

'modulation': neighbor_info['modulation'],

'data_rate': neighbor_info['data_rate'],

'beacon_rate': neighbor_info['beacon_rate'],

'channel_busy_rate': neighbor_info['channel_busy_rate']

}

# Log neighbor data to the selected options

if 'csv' in log_options and csv_writer:

csv_writer.writerow(neighbor_data)

if 'stdout' in log_options:

print(neighbor_data)

if 'socket' in log_options and socket_client:

socket_client.sendall(str(neighbor_data).encode('utf-8'))

def data_logging(net, flood_type, data_collection_interval=1, duration=60, rate_mbps=1,

dump_packets=True, log_options=None, unicast=True, check_range=True,

enable_beacon=True, beacon_rate=10, enable_cbr=False, range_detection_method='position',

ieee_standard='802.11g', run_flood=True, socket_client=None):

"""Log data of the network and perform flooding attacks."""

start_time = time.time()

pcap_file = f"{os.getcwd()}/traffic.pcap" if dump_packets else None

tcpdump_process = start_tcpdump(pcap_file) if dump_packets else None

csv_file = None

csv_writer = None

if log_options and 'csv' in log_options:

csv_file = open(f"{os.getcwd()}/network_data.csv", mode='w', newline='')

fieldnames = ['timestamp', 'car_id', 'position', 'speed', 'power_transmission', 'modulation', 'data_rate', 'beacon_rate', 'channel_busy_rate', 'neighbor_id']

csv_writer = csv.DictWriter(csv_file, fieldnames=fieldnames)

csv_writer.writeheader()

while time.time() - start_time < duration:

traci.simulationStep()

timestamp = getdatetime()

vehicles = traci.vehicle.getIDList()

for vehicle_id in vehicles:

vehicle_id_str = f'car{int(float(vehicle_id)) + 1}'

try:

car = net.getNodeByName(vehicle_id_str)

except KeyError:

continue

# Set position and speed for each car

x, y = traci.vehicle.getPosition(vehicle_id)

car.params['position'] = (x, y)

car.params['speed'] = traci.vehicle.getSpeed(vehicle_id)

power_transmission = car.wintfs[0].txpower

modulation, data_rate = get_modulation_and_datarate(car.wintfs[0].name, ieee_standard)

# Define the constants

S = 1e-10  # Sensitivity threshold in Watts

beta = 3   # Path loss exponent

m = 1      # Shape parameter

wavelength = 0.125  # Carrier wavelength in meters (e.g., 2.4 GHz corresponds to 0.125 meters)

c_d = 6    # Coded bits per OFDM symbol (example value)

b_st = 22  # Service and tail bits

n = 536 * 8  # Packet length in bits (e.g., 536 bytes)

t_ps = 40e-6  # Preamble and signal field duration in seconds (e.g., 40 microseconds)

rho = len(find_neighbors_position(car, net, check_range)) + 1  # Number of sensed neighbors including the vehicle itself

r_cs = calculate_r_cs(S, power_transmission, beta, m, wavelength)

C = calculate_C(c_d, b_st, n, t_ps)

cbr = calculate_CBR(r_cs, rho, beacon_rate, C) if enable_cbr else "N/A"

car.cbr = cbr  # Store CBR in the car object

if enable_beacon:

send_beacon(car, beacon_rate, net, check_range, range_detection_method, ieee_standard)

log_data(timestamp, car, log_options, csv_writer, socket_client)

time.sleep(data_collection_interval)

if run_flood:

flood_all(net, flood_type, rate_mbps, unicast, check_range)

if dump_packets:

stop_tcpdump(tcpdump_process)

if log_options and 'csv' in log_options and csv_file:

csv_file.close()

traci.close()

net.stop()

subprocess.run(["mn", "-c"])

sys.exit(0)

def topology(num_cars, sumo_config_file, flood_type, duration, rate_mbps, unicast=True,

check_range=True, dump_packets=True, log_options=None, enable_beacon=True,

beacon_rate=10, enable_cbr=False, range_detection_method='beacon',

ieee_standard='802.11g', run_flood=True, server_ip=None, server_port=None):

"""Set up the network topology and start the simulation."""

net = Mininet_wifi(link=wmediumd, wmediumd_mode=interference)

socket_client = None

if log_options and 'socket' in log_options and server_ip and server_port:

socket_client = setup_socket_connection(server_ip, server_port)

info("*** Creating nodes\n")

for id in range(num_cars):

car = net.addCar(f'car{id + 1}', cls=CustomCar, wlans=2, encrypt=['wpa2', ''])  # Two interfaces per car

info("*** Configuring Propagation Model\n")

net.setPropagationModel(model="logDistance", exp=2.8)

info("*** Configuring nodes\n")

net.configureNodes()

info("*** Creating Links\n")

for car in net.cars:

net.addLink(car, intf=car.wintfs[0].name, mode='g', cls=mesh, ssid='mesh-ssid', channel=1, ht_cap='HT40+')

net.addLink(car, intf=car.wintfs[1].name, mode='g', cls=mesh, ssid='mesh-ssid1', channel=6, ht_cap='HT40+')

info("*** Starting network\n")

net.build()

time.sleep(5)  # Add a delay to ensure interfaces are properly initialized

subnet_mask = calculate_subnet_mask(num_cars)

base_ip = 192 * 256**3 + 168 * 256**2

for id, car in enumerate(net.cars):

ip = base_ip + id + 1

ip_str = f'{(ip // (256**3)) % 256}.{(ip // (256**2)) % 256}.{(ip // 256) % 256}.{ip % 256}'

car.setIP(f'{ip_str}/{subnet_mask}', intf=f'{car.wintfs[0].name}')

car.setIP(f'{ip_str}/{subnet_mask}', intf=f'{car.wintfs[1].name}')

info(f"Car {car.name} assigned IP {ip_str}/{subnet_mask}\n")

info("*** Running simulation\n")

sumoCmd = ["sumo-gui", "-c", sumo_config_file, "--start", "--delay", "500"]

sumo_process = subprocess.Popen(sumoCmd)

time.sleep(2)  # Give SUMO time to start

traci.start(["sumo", "-c", sumo_config_file], port=8813)

traci.setOrder(1)

data_logging(net, flood_type, duration=duration, rate_mbps=rate_mbps,

dump_packets=dump_packets, log_options=log_options, unicast=unicast, check_range=check_range,

enable_beacon=enable_beacon, beacon_rate=beacon_rate, enable_cbr=enable_cbr,

range_detection_method=range_detection_method, ieee_standard=ieee_standard,

run_flood=run_flood, socket_client=socket_client)

info("*** Stopping network\n")

sumo_process.terminate()

if socket_client:

socket_client.close()

# Enter Mininet CLI

info("*** Running CLI\n")

CLI(net)

if __name__ == '__main__':

setLogLevel('info')

num_cars = 40  # Default number of cars

flood_type = 'ping'  # Choose from 'syn', 'ack', 'udp', or 'ping'

duration = 600  # Duration of the simulation in seconds

rate_mbps = 1  # Rate of flooding attack in Mbps

sumo_config_file = 'manhattangrid.sumocfg'  # SUMO configuration file

# Configuration flags

unicast = True  # True for unicast, False for broadcast

check_range = True  # True to check range, False to skip range check

dump_packets = True  # True to dump packets, False to skip

log_options = ['stdout', 'csv', 'socket']  # Multiple logging options allowed

server_ip = '127.0.0.1'  # IP address of the server for socket logging

server_port = 9999  # Port of the server for socket logging

enable_beacon = True  # True to enable beacon communication, False to disable

beacon_rate = 10  # Beacon rate in Hz

enable_cbr = True  # True to calculate channel busy rate, False to skip

range_detection_method = 'beacon'  # Choose from 'position' or 'beacon'

ieee_standard = '802.11g'  # Define the IEEE 802.11 standard used

run_flood = True  # Choose whether to run the flooding function

topology(num_cars, sumo_config_file, flood_type, duration, rate_mbps, unicast, check_range, dump_packets,

log_options, enable_beacon, beacon_rate, enable_cbr, range_detection_method, ieee_standard,

run_flood, server_ip, server_port)