import sys

import os

import random

import math

import time

import subprocess

from threading import Thread

# Ensure SUMO_HOME is set

if 'SUMO_HOME' not in os.environ:

os.environ['SUMO_HOME'] = "/usr/share/sumo"

sys.path.append(os.path.join(os.environ['SUMO_HOME'], 'tools'))

import pytz

import datetime

import traci

import traci.constants as tc

from mininet.log import setLogLevel, info, error

from mn_wifi.cli import CLI

from mn_wifi.net import Mininet_wifi

from mn_wifi.sumo.runner import sumo

from mn_wifi.link import wmediumd, mesh

from mn_wifi.wmediumdConnector import interference

def getdatetime():

utc_now = pytz.utc.localize(datetime.datetime.utcnow())

currentDT = utc_now.astimezone(pytz.timezone("Asia/Jakarta"))

DATIME = currentDT.strftime("%Y-%m-%d %H:%M:%S")

return DATIME

def calculate_subnet_mask(number_of_nodes):

number_of_ips_needed = number_of_nodes + 2

bits_needed_for_hosts = math.ceil(math.log2(number_of_ips_needed))

subnet_mask = 32 - bits_needed_for_hosts

return subnet_mask

def calculate_communication_range(tx_power_dbm, frequency_mhz=2400, threshold_dbm=-90):

"""

Calculate the communication range based on the transmission power using the Free Space Path Loss (FSPL) model.

:param tx_power_dbm: Transmission power in dBm.

:param frequency_mhz: Frequency in MHz (default is 2400 MHz for 2.4 GHz Wi-Fi).

:param threshold_dbm: Receive sensitivity threshold in dBm (default is -90 dBm).

:return: Communication range in meters.

"""

tx_power_watts = 10 ** (tx_power_dbm / 10) / 1000

threshold_watts = 10 ** (threshold_dbm / 10) / 1000

lambda_m = 300 / frequency_mhz

range_m = (lambda_m / (4 * math.pi)) * math.sqrt(tx_power_watts / threshold_watts)

return range_m

def is_within_reach(car1, car2):

"""Check if two cars are within the communication range of each other based on transmission power."""

x1, y1 = car1.params.get('position', (0, 0))

x2, y2 = car2.params.get('position', (0, 0))

distance = math.sqrt((x1 - x2) ** 2 + (y1 - y2) ** 2)

tx_power_car1 = car1.wintfs[0].txpower

tx_power_car2 = car2.wintfs[0].txpower

range_car1 = calculate_communication_range(tx_power_car1)

range_car2 = calculate_communication_range(tx_power_car2)

return distance <= range_car1 and distance <= range_car2

def start_http_service(car):

cmd = ["python3", "-m", "http.server", "80"]

info(f"Starting HTTP server on {car.name}\n")

subprocess.Popen(cmd, stdout=subprocess.DEVNULL, stderr=subprocess.DEVNULL, cwd="/tmp")

def start_tcpdump(pcap_file):

cmd = ["tcpdump", "-i", "any", "-w", pcap_file]

info(f"Starting tcpdump with output file {pcap_file}\n")

process = subprocess.Popen(cmd, stdout=subprocess.DEVNULL, stderr=subprocess.DEVNULL)

return process

def stop_tcpdump(process):

if process:

process.terminate()

def hping3_flood(source, target_ip, flood_type, rate_mbps=1):

cmd = [

"hping3",

"--flood",

"--rand-source",

"-p", "80",

]

if flood_type == 'syn':

cmd.append("-S")

elif flood_type == 'ack':

cmd.append("-A")

elif flood_type == 'udp':

cmd.append("--udp")

cmd.append(target_ip)

result = subprocess.run(cmd, stdout=subprocess.DEVNULL, stderr=subprocess.DEVNULL)

if result.returncode == 0:

info(f"Successfully sent {flood_type.upper()} flood from {source.name} to {target_ip}")

def ping_flood(source, target_ip):

cmd = ["ping", "-i 0.01  ", target_ip]

result = subprocess.run(cmd, stdout=subprocess.DEVNULL, stderr=subprocess.DEVNULL)

if result.returncode == 0:

info(f"Successfully sent ping flood from {source.name} to {target_ip}")

def flood_all(net, flood_type, rate_mbps=1):

def flood_car(car1, car2):

target_ip = car2.params['ip']

if flood_type == 'ping':

ping_flood(car1, target_ip)

else:

hping3_flood(car1, target_ip, flood_type, rate_mbps)

threads = []

for car1 in net.cars:

for car2 in net.cars:

if car1 != car2 and is_within_reach(car1, car2):

thread = Thread(target=flood_car, args=(car1, car2))

threads.append(thread)

thread.start()

for thread in threads:

thread.join()

def data_logging(net, flood_type, data_collection_interval=1, duration=60, rate_mbps=1):

start_time = time.time()

pcap_file = f"{os.getcwd()}/traffic.pcap"

tcpdump_process = start_tcpdump(pcap_file)

total_packets_sent = 0

while time.time() - start_time < duration:

traci.simulationStep()

timestamp = getdatetime()

vehicles = traci.vehicle.getIDList()

for vehicle_id in vehicles:

vehicle_id_str = f'car{int(float(vehicle_id)) + 1}'

x, y = traci.vehicle.getPosition(vehicle_id)

coord = [x, y]

lon, lat = traci.simulation.convertGeo(x, y)

gpscoord = [lon, lat]

spd = round(traci.vehicle.getSpeed(vehicle_id) * 3.6, 2)

edge = traci.vehicle.getRoadID(vehicle_id)

lane = traci.vehicle.getLaneID(vehicle_id)

displacement = round(traci.vehicle.getDistance(vehicle_id), 2)

turnAngle = round(traci.vehicle.getAngle(vehicle_id), 2)

nextTLS = traci.vehicle.getNextTLS(vehicle_id)

try:

car = net.getNodeByName(vehicle_id_str)

car.params['position'] = (x, y)

except KeyError:

continue

power_transmission = car.wintfs[0].txpower

modulation = "N/A"

veh_data = {

'timestamp': timestamp,

'vehicle_id': vehicle_id_str,

'position': coord,

'gps_position': gpscoord,

'speed_kmh': spd,

'edge_id': edge,

'lane_id': lane,

'distance_m': displacement,

'angle_deg': turnAngle,

'next_traffic_light': nextTLS,

'power_transmission': power_transmission,

'modulation': modulation,

}

print(veh_data)

time.sleep(data_collection_interval)

flood_all(net, flood_type, rate_mbps)

total_packets_sent += 1

stop_tcpdump(tcpdump_process)

info(f"Total packets sent: {total_packets_sent}")

traci.close()

net.stop()

subprocess.run(["mn", "-c"])

sys.exit(0)

def topology(num_cars, sumo_config_file, flood_type, duration, rate_mbps):

net = Mininet_wifi(link=wmediumd, wmediumd_mode=interference)

info("*** Creating nodes\n")

for id in range(num_cars):

car = net.addCar(f'car{id + 1}', wlans=1, encrypt=['wpa2', ''])  # One interface per car

Thread(target=start_http_service, args=(car,)).start()

info("*** Configuring Propagation Model\n")

net.setPropagationModel(model="logDistance", exp=2.8)

info("*** Configuring nodes\n")

net.configureNodes()

for car in net.cars:

net.addLink(car, intf=car.wintfs[0].name, cls=mesh, ssid='mesh-ssid', channel=5, ht_cap='HT40+')  # Mesh link with one interface

info("*** Starting network\n")

net.build()

subnet_mask = calculate_subnet_mask(num_cars)

base_ip = 192 * 256**3 + 168 * 256**2

for id, car in enumerate(net.cars):

ip = base_ip + id + 1

ip_str = f'{(ip // (256**3)) % 256}.{(ip // (256**2)) % 256}.{(ip // 256) % 256}.{ip % 256}'

car.setIP(f'{ip_str}/{subnet_mask}', intf=f'{car.wintfs[0].name}')

info(f"Car {car.name} assigned IP {ip_str}/{subnet_mask}\n")

info("*** Running simulation\n")

sumoCmd = ["sumo-gui", "-c", sumo_config_file, "--start", "--delay", "500"]

sumo_process = subprocess.Popen(sumoCmd)

time.sleep(2)  # Give SUMO time to start

traci.start(["sumo", "-c", sumo_config_file], port=8813)

traci.setOrder(1)

data_logging(net, flood_type, duration=duration, rate_mbps=rate_mbps)

info("*** Stopping network\n")

sumo_process.terminate()

# Enter Mininet CLI

info("*** Running CLI\n")

CLI(net)

if __name__ == '__main__':

setLogLevel('info')

num_cars = 100  # number of cars

flood_type = 'syn'  # Choose from 'syn', 'ack', 'udp', or 'ping'

duration = 60  # Duration of the simulation in seconds

rate_mbps = 1  # Rate of flooding attack in Mbps

sumo_config_file = '6lane.sumocfg'  # SUMO configuration file

topology(num_cars, sumo_config_file, flood_type, duration, rate_mbps)